# ENDCAP Alignment

6U LOW VOLTAGE DISTRIBUTOR

##### User’s Manual

FNAL – PNPI

2005

##### Introduction

The low voltage distributor provides low voltage supplying of DCOPS or analog interface boards. There are no active components at the board. No control is required for this board.

###### Construction

The low voltage distributor is 6U 8HP VME module without backplane connectors. Its printed circuit board has size C (233 mm height x 340 mm depth).

The low voltage distributor has seven connectors:

J1	– 6-cont. Molex Sabre header 43160-3106 (Low Voltage Input)

J2, J3	– 6-cont. Molex SL header 70553-0005 (Low Voltage Output)

J4–J7	– 2-cont. Molex Sabre header 43160-3102 (Low Voltage Output)

- Circuit Diagram

There are no active components at the board. The board provides interconnections between connectors only. Neither debugging nor test is required.

#### Functioning

## This module is intended for low voltage distribution for both DCOPS and analog electronics.

## When it is used for DCOPS low voltage distribution two voltages are connected to the input connector J1: D15V and F7V. In this case the output connectors J2, J3 provide supplying of RS-422 decouplers; the output connectors J4 – J7 should be connected to DCOPS interface boards #1.

## When it is used for analog low voltage distribution three voltages are connected to the input connector J1: A12V, D7V, and C7V. In this case the output connectors J2, J3 provide supplying of analog readout modules; the output connectors J4 – J7 should be connected to analog interface boards.

- Specification

Dimensions of the board 	13.386" x 9.187"

The module does not consume power.

Specification of connectors

Connector J1 (LV Input):

DCOPS LV				Analog LV

| Pin | Name |  | Pin | Name |
| --- | --- | --- | --- | --- |
| 1 | D15V |  | 1 | A12V |
| 2 | GND |  | 2 | AGND |
| 3 | F7V |  | 3 | D7V |
| 4 | FGND |  | 4 | GND |
| 5 | - |  | 5 | C7V |
| 6 | - |  | 6 | CGND |

Connectors J2, J3 (LV Output):

DCOPS LV				Analog LV

| Pin | Name |  | Pin | Name |
| --- | --- | --- | --- | --- |
| 1 | D15V |  | 1 | A12V |
| 2 | GND |  | 2 | AGND |
| 3 | F7V |  | 3 | D7V |
| 4 | FGND |  | 4 | GND |
| 5 | - |  | 5 | C7V |
| 6 | - |  | 6 | CGND |

Connectors J4-J7 (LV Output):

DCOPS LV				Analog LV

| Pin | Name |  | Pin | Name |
| --- | --- | --- | --- | --- |
| 1 | D15V |  | 1 | A12V |
| 2 | GND |  | 2 | AGND |

Fig. 1. 6U Low Voltage Distributor